TC TIMER BLE Module to Smartphone BLE Communication

Smart Phone

BLE

TC Timer

BLE

## TC Timer Information

The Timer has 499 memory locations:

- Location 0 is unused.

- The first time is saved to location 1, second time to location 2, etc.

- When the memory is full, location 499 is continually overwritten.

- Clearing the memory starts over at location 1.

Each memory location holds the following:

- Athlete Number - the number assigned to an athlete for a given session

- Test ID #

- Flag bits

- Running Time in ms.

- Up to 20 split times in ms (relative to Start Time)

- (2 additional Appendix packets may follow with up to 20 split times)

- The last non zero split time is considered the Finish time

## BLE Services and Characteristics

Service UUID

- UUID: 11574949-d37a-4fd7-a171-f36fcdc3a461

- four characteristics:

  - Athlete Data Characteristic: Use for transferring athlete data including bib number, start time, finish time, and optional split times.

  - Time Base Characteristic: Use for reading the timer's base time (ms since power on)

  - Power On Counter Characteristic

  - Current Memory Location Characteristic

## Athlete Data Characteristic

UUID: bb8722a4-3810-4fd3-9321-f6cbdcc1ea4e

Request athlete data by writing a command to this characteristic (see command format below). Results will be delivered through this characteristic as Notifications.

### Command Types:

| Type Description | Value (hex) |
| --- | --- |
| Athlete Data | 0x01 |
| Time Base, Session Number, Sequence number | 0x02 |
| Freq Channel Number | 0x03 |
| Test | 0x04 |
| New Athlete (same as pressing “new” button on the TC Timer) | 0x0A |

### Command format (7 bytes)

The Smartphone sends the following format to retrieve data in a specific memory range. If the range extends into unused memory locations, only the used locations will be delivered.

Each packet can be sent as fast as 25 ms.

| Byte index | Description | Value (hex) |
| --- | --- | --- |
| 0 | Command Start byte | 0x55 |
| 1 | Type | 0x01 for Athlete Data |
| 2-3 | Start memory location (uint16) | Example: 0x0004 |
| 4-5 | Stop memory location (uint16) | Example: 0x0006 |
| 6 | Command Stop byte | 0xAA |

### New Athlete Command format

| Byte index | Description | Value (hex) |
| --- | --- | --- |
| 0 | Command Start byte | 0x55 |
| 1 | Type | 0x0A for New Athlete |
| 2-3 | Athlete Number | Example: 0x0004 |
| 4-5 | 0000 | 0000 |
| 6 | Command Stop byte | 0xAA |

MSP UART Response data to BLE Module

### Athlete Data format

TC Timer can have 1,2 or 3 packets per Athlete.

Split Packet Format (20 bytes)

| Byte index | Byte Size |  |
| --- | --- | --- |
| 0 | 1.5 | Memory location |
| 1.5 | 1.5 | Athlete Number |
| 3 | 1 | Test ID # |
| 4 | 2 | Unused Flags |
| 6 | 3.5 | Start Time (absolute) |
| 9.5 | 2.5 | Split 1 (relative to start time) |
| 12 | 2.5 | Split 2 (relative to start time) |
| 14.5 | 2.5 | Split 3 (relative to start time) |
| 17 | 2.5 | Split 4 (relative to start time) |
| 19.5 | 0.5 | Number of Split Appendix Packets(0,1,2) |

Apendix1 Packet Format (20 bytes)

| Byte index | Byte Size | Description |
| --- | --- | --- |
| 0 | 2.5 | Split 5 (relative to start time) |
| 2.5 | 2.5 | Split 6 (relative to start time) |
| 5 | 2.5 | Split 7 (relative to start time) |
| 7.5 | 2.5 | Split 8 (relative to start time) |
| 10 | 2.5 | Split 9 (relative to start time) |
| 12.5 | 2.5 | Split 10 (relative to start time) |
| 15 | 2.5 | Split 11 (relative to start time) |
| 17.5 | 2.5 | Split 12 (relative to start time) |

Apendix2 Packet Format (20 bytes)

| Byte index | Byte Size | Description |
| --- | --- | --- |
| 0 | 2.5 | Split 13 (relative to start time) |
| 2.5 | 2.5 | Split 14 (relative to start time) |
| 5 | 2.5 | Split 15 (relative to start time) |
| 7.5 | 2.5 | Split 16 (relative to start time) |
| 10 | 2.5 | Split 17 (relative to start time) |
| 12.5 | 2.5 | Split 18 (relative to start time) |
| 15 | 2.5 | Split 19 (relative to start time) |
| 17.5 | 2.5 | Split 20 (relative to start time) |

Athlete data has bib # 1000

- Session headers are stored as athletes with bib number 1000

- Session # data is sent as a 20 byte packet

| Byte index | Byte Size | Description |
| --- | --- | --- |
| 0 | 1.5 | Memory location |
| 1.5 | 1.5 | Athlete Number 1000 |
| 3 | 3.5 | Session start time |
| 6.5 | 3.0 | Session number |
| 9.5 | .5 | 0 |

### Time, Session, Sequence Data Return format

| Co | Byte Size | Description |
| --- | --- | --- |
| 0 | 4 | time base in ms |
| 4 | 2 | Power on Counter / power on Session # |
| 6 | 2 | Sequence number of current Session |
| 8 | 12 | Not Used |

Time Base Characteristic

UUID: ee152c14-79a7-447d-b435-030880aa7d7d

Readable characteristic, returns current time since the TC timer has been powered on. This is the time base from which all athlete time of day is calculated. (4 bytes, ms since power on)

Time Base Packet (4 bytes)

| Byte index | Byte Size | Description |
| --- | --- | --- |
| 0 | 4 | time base in ms |

Power On Counter Characteristic, Also known as current session number

UUID: 882c254a-d1f1-440f-8d08-4d0e5a9d4226

Readable characteristic, returns the session number from when the TC Timer was last powered on or cleared. This is used to determine what sessions have a valid time base. If the TC Timer memory is cleared the new bib 1000 packet referenced by this characteristic will have a non zero start time.

Power On Counter Packet (16 bytes)

| Byte index | Byte Size | Description |
| --- | --- | --- |
| 0 | 2 | Power on Counter / power on Session # |
| 2 | 2 | memory location of current Session |
| 4 | 12 | Not Used |

## How to calculate Session Start "time of day" and Athlete "time of day"

Read UUID: ee152c14-79a7-447d-b435-030880aa7d7d

- The returned value is the TC Timer's time base in milliseconds.

- The time base is the number of milliseconds since the TC Timer was powered on.

- Use the Smartphone's clock and subtract the time base to get the clock time when the TC Timer was powered on.

- The result is used in the session menu

- It is possible to calculate the time of day for each athlete referenced to the smartphone's clock.

## Serial Number Characteristic

Service UUID: ffae864c-ee9f-4f31-ad8a-9bcbac855a9f

- One characteristic:

- UUID: beef94d7-4124-423d-82e0-5aa556bc722b

- Serial Number Characteristic: Use to uniquely identify TC Timer units.

## How to round time data

- Time data is in milliseconds (.001) and displayed in 100ths of a second (.01)

- Truncate the last digit to get the displayed time, do not round to the nearest 100th.

- 19.999 becomes 19.99 not 20.00

## Dynamic advertising data

** BLE Advertising Packet (30 bytes)**

ie: 02 01 06 11 07 daefdfe3f6b8659e77473ac91e2de9aa 08 ff ffff 01ab 0053 04

| Byte index | Byte Size | hex Value | Description |
| --- | --- | --- | --- |
| 0 | 1 | 02 | AD field length = 2 bytes |
| 1 | 1 | 01 | AD field type= flags |
| 2 | 1 | 06 | flags = connectable/undirected |
| 3 | 1 | 11 | AD field length = 17 bytes |
| 4 | 1 | 07 | ad field type =characteristic name |
| 5 | 16 |  | Characteristic name |
| 21 | 1 | 08 | AD field length = 8 bytes |
| 22 | 1 | FF | AD field type= manuf. spacific |
| 23 | 2 | FFFF | Prototype ID |
| 25 | 2 |  | Session number |
| 27 | 2 |  | Current memory location |
| 29 | 1 |  | TBD |